STORE NAME:				______	DODAAC:_________________

(Store name and DODAAC number to be completed when submitted to DOLM)

STORE REQUIREMENT:  _____ each

UNIT OF PACK (UPK):  1 (one) each.

- Purpose of Equipment:  To provide a tool which allows a single user the ability to burnish a commissary sales floor.

- Generalized Operating Specifications:

  - Dimensions:

    - 25 inches long, 20 inches wide, 45 inches high, 20 inch pad size.  Dimensional sizes, such as length, width, and height are not critical to this equipment so long as the burnishing pad size is met.

  - Construction:

    - Commercial off the shelf (COTS) standard unit, no unique DeCA requirements.

  - Operating Characteristics that shall be provided:

# Expected life span of this equipment is required to be 6 years or more with basic maintenance.

    - Deck constructed of heavy duty polyethylene.

    - Adjustable handle constructed of aluminum.

    - Minimum 1.5 horsepower motor.

    - Integrated brush wear indicator.

    - Dust filter to protect motor.

    - Dust shroud with heavy duty felt seals to the floor.

    - Minimum pad speed of 2000 Revolutions Per Minute (RPM).

    - Self-regulating pad pressure for constant full contact with the floor.

    - Squeeze grip levers control the power start / stop.

    - One case of 5 burnishing pads.

  - Networking / Communications that shall be provided:  N / A

- Operating Options:  N / A

- Electrical Requirements:

# 115 volts, 60 Hertz, 1 phase, with an average amperage draw of 15 amps.

  - Minimum 70 foot cord with a NEMA 5-15P plug.

- Color Requirements:  N / A

- Maintenance Requirements:  Continued parts support for equipment is required for the projected total life cycle of the equipment plus 5 years.

- Industry Standards and Requirements:

  - Industry Standards:  Underwriters Laboratory (UL) listed or equivalent.

  - Contractor Requirements:

    - Must be shipped ready to assemble, plug in and use.

    - No additional work shall be required on the part of the government.

# Special / Specific Equipment Safety Requirements:  N / A

- Information To Be Provided By The Contractor To The Commissary At Time of Delivery:

  - Commercial Warranty and Point of Contact for Warranty Service.

  - Installation and Operating Instructions.

  - Parts List.

# Federal Supply Class:  7910

- Special Considerations For Installation/De-installation Of Equipment: (NOTICE/WARNING)

  - Contractor:  N / A

  - Store:  This item should be on the store Commercial Activity (CA) custodial contract and the work performed by a DeCA contractor.

- Basis of Allocation:  Justification for this equipment item needs to be provided by the store and proof there is not a CA contract in place.  If approved 1 per small to medium stores, two per large stores.

- Recommended Source(s) or equal:

  - Clarke, Model number Ultra Speed Series Burnisher US 2000 DC.

  - Nilfisk-Advance, Model number Whirlamatic 20 Ultra.

- Store Request for Specific Model to Match Existing Equipment:  N / A

- Store Maintenance Requirements:  The purpose of maintenance planning is to sustain equipment operations post purchase, to identify maintenance requirements, to reduce maintenance costs, and to extend the life cycle of the equipment.

  - Within Warranty Period:

    - Consult manufacturer’s warranty documentation and contact instructions.

  - After Warranty Period:

    - Use a one time service call with store GPC as required