STORE NAME:				______	DODAAC:_________________

(Store name and DODAAC number to be completed when submitted to DOLM)

STORE REQUIREMENT:  _____ each

UNIT OF PACK (UPK):  1 (one) each.

- Purpose of Equipment:  To provide a tool which allows a single user the ability to scrub brush a commissary sales floor.

- Generalized Operating Specifications:

  - Dimensions:

    - 55 inches long, 25 inches wide, 40 inches high, 26 inch brush (es) diameter.  Dimensional sizes, such as length, width, and height are not critical to this equipment as long as the brush size is met.

  - Construction:

    - Commercial off the shelf (COTS) standard unit, no unique DeCA requirements.

  - Operating Characteristics that shall be provided:

# Expected life span of this equipment is required to be 6 years or more with basic maintenance.

    - Frame constructed of steel.

    - Body constructed of heavy-duty polyethylene.

    - Minimum 0.75 horsepower (HP) vacuum motor.

    - Minimum 0.5 HP brush motor.

    - Minimum 0.4 HP propulsion motor.

    - Variable speeds, forward and reverse.

    - Disc type scrub brush (es).  Can be equipped with either 1 or 2 brushes.

    - Minimum 200 Revolutions per Minute (RPM) disc rotation speed.

    - Minimum 20 gallon cleaning solution and recovery tank.

    - Fingertip controls for cleaning solution flow rate and brush pressure.

    - Minimum squeegee width of 32 inches.

    - Squeegee breaks away to prevent damage to store fixtures.

    - Vacuum operates automatically when squeegee is lowered to floor.

    - Non-marking vinyl or urethane drive wheels.

  - Networking / Communications that shall be provided:  N / A

- Operating Options:  N / A

- Electrical Requirements:

# 24 volts, (4) 6 volt batteries with a minimum 195 Ampere-Hour (AH).

  - Battery charger:  115 volts, 60 Hertz input / 36 volts, 25 amps output.

- Color Requirements:  N / A

- Maintenance Requirements:  Continued parts support for equipment is required for the projected total life cycle of the equipment plus 5 years.

- Industry Standards and Requirements:

  - Industry Standards:  Underwriters Laboratory (UL) listed or equivalent.

  - Contractor Requirements:

    - Must be shipped ready to assemble, plug in and use.

    - No additional work shall be required on the part of the government.

# Special / Specific Equipment Safety Requirements:  N / A

- Information To Be Provided By The Contractor To The Commissary At Time of Delivery:

  - Commercial Warranty and Point of Contact for Warranty Service.

  - Installation and Operating Instructions.

  - Parts List.

# Federal Supply Class:  7910

- Special Considerations For Installation/De-installation Of Equipment: (NOTICE/WARNING)

  - Contractor:  N / A

  - Store:  This item should be on the store Commercial Activity (CA) custodial contract and the work performed by a DeCA contractor.

- Basis of Allocation:  1 unit for small to medium stores, two per larger stores.  This unit is intended for cleaning areas not covered by the CA contract.  Additional units can be requested through the Maintenance & Replacement Branch (DOLM).  The store must provide additional information regarding coverage areas of the CA contract or the lack there of.

- Recommended Source(s) or equal:

  - Advance, Model number Convertamatic 26D.

  - Clarke, Model number Encore 24 / 26 Midsize Series.

- Store Request for Specific Model to Match Existing Equipment:  N / A

- Store Maintenance Requirements:  The purpose of maintenance planning is to sustain equipment operations post purchase, to identify maintenance requirements, to reduce maintenance costs, and to extend the life cycle of the equipment.

  - Within Warranty Period:

    - Consult manufacturer’s warranty documentation and contact instructions.

  - After Warranty Period:

    - Use a one time service call with store GPC as required.