STORE NAME:				______	DODAAC:_________________

(Store name and DODAAC number to be completed when submitted to DOLM)

STORE REQUIREMENT:  _____ each

UNIT OF PACK (UPK): 1 (one) each

- Purpose of Equipment:  To move palletized cargo loads from ground level to elevated storage areas.

- Generalized Operating Specifications

  - Dimensions:

    - 75 inches long, 37 inches wide, 83 inches high, not to exceed 83 inches in height.

  - Construction:

    - Commercial off the shelf (COTS) standard unit, no unique DeCA requirements.

  - Operating Characteristics that shall be provided:

# Expected life span of this equipment is required to be 12 years or more with basic maintenance.

    - Minimum operational load rated capacity of 4000 pounds.

    - Power steering.

# Fork length is 42 inches unless otherwise specified below.

# Collapsed mast unit; height not to exceed 83 inches.

# Standard elevated fork height to be 187 inches (Triplex) at full extension.

# Four wheeled unit.

# Non-marking rubber cushioned wheels for indoor/improved surface operations.

    - Wet charged battery provided for CONUS shipment and dry charged battery for OCONUS shipment.

    - Horn, key switch, hour meter, and discharge indicator.

    - Zero degree cold storage / freezer conditioning package standard for all OCONUS shipments.

    - Anderson SB350 gray electrical connector.

# 36 volt electrical system.

    - Battery rated for 8 hours of continuous duty.

# Industrial battery with minimum 680 ampere hour capacity.

  - Networking / Communications that shall be provided: N / A

- Operating Options:

  - Standard fork tine length is 42 inches.  Optional fork lengths are:                        ____ 36 inches, ____ 48 inches.

  - ____ Charger required (unless this line is checked, DO NOT SEND charger).

    - ____ CONUS, 60 Hz model required.

    - ____ OCONUS, 50 Hz model required.

  - ____ Cold storage option, -20 degrees cold storage package.

  - ____ Solid, lugged tires for slightly better traction on slick surfaces required.

- Electrical Requirements: Optional battery charger (CED 1R04), standard CONUS electrical requirement for charger is to be 208-480V / 60 Hz / 3 Ph (not to exceed 20 amps  at 480V, 30 amps per 208-240 V).  Charger to be factory set to 480 volts with the capability for adjustment at the store by an electrician as needed.

- Color Requirements:  N / A

- Maintenance Requirements: Equipment contractor must have the means or support organizations in place to perform repairs on equipment.  This support is to include supervision, tools, manuals/prints, skilled labor, transportation, and parts.

  - Sustainability:  Continued support for equipment is required for the projected total life cycle of the equipment plus 5 years.

- Industry Standards and Requirements:

  - Industry Standards: Underwriters Laboratory (UL) listed, or equivalent.

  - Contractor Requirements:

    - Must be shipped ready to be charged and used.

    - No additional work shall be required on the part of the government.

# Special / Specific Equipment Safety Requirements:

# Provided with a fire extinguisher.

# Provided with a back up alarm.

# Overhead protective guard over the operator.

- Information To Be Provided By The Contractor To The Commissary At Time of Delivery:

  - Commercial Warranty and Point of Contact for Warranty Service.

  - Installation and Operating Instructions.

  - Parts List.

# Federal Supply Class: 3930

- Special Considerations For Installation/De-installation Of Equipment: (NOTICE/WARNING)

  - Contractor:  N / A

  - Store:  A safety platform must be used (CED 2R11) when lifting people with forklift

# Basis of Allocation:  K-1A and K-1 (one each), K-2 and K-3 (two each), K-4 (four each), K-5 (six each), the amount allowed per store is not a requirement, a store can have less.

- Recommended Source(s) or equal:

  - DOD, Defense Logistics Agency (DLA) contract is DeCA’s source for MHE.

  - Yale, Model ERC040AHN36TE082

- Store Request for Specific Model to Match Existing Equipment:  N / A

- Store Maintenance Requirements:  The purpose of maintenance planning is to sustain equipment operations post purchase, to identify maintenance requirements, to reduce maintenance costs, and to extend the life cycle of the equipment.

  - Within Warranty Period:

    - Consult manufacturer’s warranty documentation and contact instructions.

  - After Warranty Period:

    - Have unit added to the MHE/Baler maintenance service contract.

    - It is normal for the battery to be replaced up to two times during the life cycle of this equipment.