STORE NAME:				______	DODAAC:_________________

(Store name and DODAAC number to be completed when submitted to DOLM)

STORE REQUIREMENT:  _____  each

UNIT OF PACK (UPK):  1 (one) each

- Purpose of Equipment:  To facilitate palletized cargo movement and order picking in narrow aisles (72 inches) from ground level to elevated storage.

- Generalized Operating Specifications:

  - Dimensions:

    - 121 inches long from back to fork tips, 41 inches wide, 99 inches high, not to exceed 89 inches in height.

  - Construction:

    - Commercial off the shelf (COTS) standard unit, no unique DeCA requirements.

  - Operating Characteristics that shall be provided:

# Expected life span of this equipment is required to be 12 years or more with scheduled preventative maintenance.

    - Minimum operational load capacity of 3,000 pounds.

    - Power steering.

    - Fork length is 42 inches unless otherwise specified in Operating Options.

    - Collapsed mast unit; height not to exceed 89 inches (to include the overhead guard).

    - Standard elevated fork height to be 195 inches (Triplex) at full extension.

    - Four wheeled unit.

    - Non-marking rubber or polyurethane wheels for indoor / improved surface.

    - Fire extinguisher.

    - Backup alarm and strobe lights.

    - Overhead protective guard over the operator.

    - Tether and safety harness.

    - Anderson SB175 red battery connectors.

    - Minimum 24 volt operating system.

    - Horn, dual spotlights, key switch, hour meter and discharge indicator.

    - Pallet clamp and centering device.

    - Cold storage / freezer conditioning option to be provided when specified in Operating Options.

    - Wet charge battery provided for CONUS (Continental USA) shipment.   Dry charge battery for OCONUS (Overseas) shipment.

  - Networking / Communications that shall be provided:  N / A

- Operating Options:

  - Standard fork length is 42 inches.  Optional fork lengths are:

    - _____  36 inches.

    - _____  48 inches.

  - _____ Cold storage option, -20 degree cold storage package required.

  - _____Charger required (unless this line is checked, DO NOT SEND charger).

3.2.1____ CONUS, 60 Hz model required.

3.2.2____ OCONUS, 50 Hz model required.

- Electrical Requirements:  Optional battery charger (CED 1R01), standard CONUS electrical requirements for charger is to be 208-480V / 60 Hz / 3 Ph (not to exceed 20 amps at 480V, 30 at per 208-240V). Charger to be factory set at 480 volts with the capability for adjustment at the store by an electrician as needed.

- Color Requirements:  N / A

- Maintenance Requirements:  Continued parts support for equipment is required for the projected total life cycle of the equipment plus 5 years.

- Industry Standards and Contractor Requirements:

  - Industry Standards: Underwriters Laboratory (UL) listed, or equivalent.

  - Contractor Requirements:

    - Must be shipped ready to charge and use.

    - No additional work shall be required on the part of the government.

# Special / Specific Equipment Safety Requirements:  N / A

- Information To Be Provided By The Contractor To The Commissary At Time of Delivery:

  - Commercial Warranty and Point of Contact for Warranty Service.

  - Installation and Operating Instructions.

  - Parts List.

# Federal Supply Class:  3930

- Special Considerations For Installation/De-installation Of Equipment:

11.1    Contractor:  N / A

11.2    Store:   A safety platform (CED 2R11) must be used when lifting people with a

forklift.

- Basis of Allocation:  Authorized only for hub stores supporting several satellite stores and  CDC’s where elevated picking operations are required. Quantity usually determined by warehouse operations space and rack layout. Two each for a hub store.

- Recommended Source(s) or equal:

  - Yale, Model OS030ECN24TE089.

- Store Request for Specific Model to Match Existing Equipment:  N / A

- Store Maintenance Requirements:  The purpose of maintenance planning is to sustain equipment operations post purchase, to identify maintenance requirements, to reduce maintenance costs, and to extend the life cycle of the equipment.

  - Within Warranty Period:

    - Consult manufacturer’s warranty documentation and contact instructions.

  - After Warranty Period:

    - Have unit added to the MHE maintenance service contract.

    - NOTE: It is normal for the battery to be replaced up to two times during the life cycle of this equipment.