STORE NAME:				______	DODAAC:_________________

(Store name and DODAAC number to be completed when submitted to DOLM)

STORE REQUIREMENT:  _____ (ea)

UNIT OF PACK (UPK):  1 (one) each

- Purpose of Equipment:   To sweep and scrub smooth floor surfaces inside the warehouse, loading docks and entrance areas.

- Generalized Operating Specifications:

  - Dimensions:

    - 87 inches long, 46 inches wide and 52 inches high.  Dimensional sizes, such as length, width, and height are not critical so long as it is a rider model.

  - Construction:

    - Commercial off the shelf (COTS) standard unit, no unique DeCA requirements.

  - Operating Characteristics that shall be provided:

# Expected life span of this equipment is required to be 8 years or more with basic maintenance.

    - 36 volt operation.

    - Anderson SB350-gray battery connectors.

    - Battery charger.

    - 46 inch cleaning path.

    - 55 gallon solution and recovery tanks for scrubber.

    - 3 each 16 inch scrubbing brushes.

    - Power steering.

    - Electric horn, ignition lock and hour meter.

    - Sweeper / Scrubber and charger to be prepped for overseas shipment when indicated below.

    - Break away squeegee.

    - Dust control option.

  - Networking / Communications that shall be provided:   N / A

- Operating Options: _____Must be prepped for overseas shipment.

- Electrical Requirements:

  - Charger to be field adjustable from 208, 240 or 480 volts.

# U.S.A. CONUS stores to be factory default set at 480 volts, 60 Hertz, 3 phase.

  - _____ Pacific overseas store requesting 208V / 50HZ / 1PH.

  - _____ Pacific overseas store requesting 208V / 60HZ / 1PH.

  - _____ Europe overseas store requesting 220V / 50HZ / 1PH.

- Color Requirements:   N / A

- Maintenance Requirements:  Continued parts support for equipment is required for the projected total life cycle of the equipment plus 5 years.

- Industry Standards and Contractor Requirements:

  - Industry Standards: Underwriters Laboratory (UL) listed.

  - Contractor Requirements:

    - Must be shipped ready to make utility connections, plug in, and use.

    - No additional work shall be required on the part of the government.

# Special / Specific Equipment Safety Requirements:   N / A

- Information To Be Provided By The Contractor To The Commissary At Time of Delivery:

  - Commercial Warranty and Point of Contact for Warranty Service.

  - Installation and Operating Instructions.

  - Parts List.

# Federal Supply Class:   7910

- Special Considerations For Installation/De-installation Of Equipment:

  - Contractor:  N / A

  - Store:

    - Battery charger to be installed by a licensed electrician.

    - Sweeper / scrubber unit to be serviced and placed into operation by local

supporting maintenance activity.

- Basis of Allocation:  1 per CDC.

- Recommended Source(s) or equal:

  - American-Lincoln,  Model SC7730

- Store Request for Specific Model to Match Existing Equipment:  N / A

- Store Maintenance Requirements:  The purpose of maintenance planning is to sustain equipment operations post purchase, to identify maintenance requirements, to reduce maintenance costs, and to extend the life cycle of the equipment.

  - Within Warranty Period:

    - Consult manufacturer’s warranty documentation and contact instructions.

  - After Warranty Period:

    - Have unit added to the MHE maintenance service contract.

    - It is normal for the battery to be replaced during the life cycle of this equipment.