STORE NAME:				______	DODAAC:_________________

(Store name and DODAAC number to be completed when submitted to DOLM)

STORE REQUIREMENT:  _____ each

UNIT OF PACK (UPK):  1 (one) each.

- Purpose of Equipment:  To allow the safety pallet occupant access to elevated areas to perform tasks such as changing ceiling light bulbs or hanging seasonal decorations.

- Generalized Operating Specifications:

  - Dimensions:

    - 48 inches long, 36 inches wide, 60 inches high. Dimensional sizes, such as length, width, and height are not critical to this equipment so long as this equipment is a steel framed safety pallet.

  - Construction:

    - Commercial off the shelf (COTS) standard unit, no unique DeCA requirements.

  - Operating Characteristics that shall be provided:

# Expected life span of this equipment is required to be 10 years or more with basic maintenance.

    - Frame to be constructed of steel.

    - Platform to be covered with a skid-resistant mat.

    - Platform to have 40 inch high guard rails and entry gate.

    - Four inch toe boards to prevent tools/equipment from falling off platform.

    - Wire mesh screen at rear of platform to protect occupant from forklift.

    - Full length fork pockets.

    - Chain to secure platform to forklift mast.

    - Minimum 500 pound carrying capacity.

    - Casters.

  - Networking / Communications that shall be provided:  N / A

- Operating Options:  N / A

- Electrical Requirements:  N / A

- Color Requirements:  N / A

- Maintenance Requirements:

  - Sustainability:  Continued parts support for equipment is required for the projected total life cycle of the equipment plus 5 years.

- Industry Standards and Requirements:  N / A

  - Contractor Requirements:

    - Must be shipped ready to assemble and use.

    - No additional work shall be required on the part of the government.

# Special / Specific Equipment Safety Requirements:

# The forklift should not be driven when safety pallet is elevated.

- Information To Be Provided By The Contractor To The Commissary At Time of Delivery:

  - Commercial Warranty and Point of Contact for Warranty Service.

  - Installation and Operating Instructions.

  - Parts List.

# Federal Supply Class:  3930

- Special Considerations For Installation/De-installation Of Equipment:  N / A (NOTICE/WARNING)

- Basis of Allocation:  One per store; most stores can order this item instead of the aerial scissors lift, CED# 1R15-A/B.

- Recommended Source(s) or equal:

  - Ballymore, Model FD-48-C.

- Store Request for Specific Model to Match Existing Equipment:  N / A

- Store Maintenance Requirements:  The purpose of maintenance planning is to sustain equipment operations post purchase, to identify maintenance requirements, to reduce maintenance costs, and to extend the life cycle of the equipment.

  - Within Warranty Period:

    - Consult manufacturer’s warranty documentation and contact instructions.

  - After Warranty Period:

    - Self-repair or replace the unit as required.