STORE NAME:				______	DODAAC:_________________

(Store name and DODAAC number to be completed when submitted to DOLM)

STORE REQUIREMENT:  _____ (ea)

UNIT OF PACK (UPK):  1 (one) each

- Purpose of Equipment:  To provide a sink with a wash, rinse and sanitize basin for food preparation areas to meet sanitation requirements.

- Generalized Operating Specifications:

  - Dimensions:

    - 4S03-L:  Sink compartment with attached drain board to the left (as you stand in front of the sink).  Overall length is approximately 107 inches by 31 inches deep.

    - 4S03-D:  Sink compartment with attached drain boards (right and left). Overall length is approximately 127 inches by 31 inches deep.

    - 4S03-R:  Sink compartment with attached drain board to the right (as you stand in front of the sink).  Overall length is approximately 107 inches by 31 inches deep.

    - Basin:  Minimum 24 inches by 24 inches square (coved corners), 14 inches deep.  Dimensional sizes may vary plus or minus 1 inch.

    - Drain Board: Minimum 24 inches long by 31 inches wide. Dimensional size may vary by plus or minus 2 inches.

  - Construction:

    - Commercial off the shelf (COTS) standard unit, no unique DeCA requirements.

  - Operating Characteristics that shall be provided:

# Expected life span of this equipment is required to be 12 years or more with basic maintenance.

# Sink:

# Sink bowl compartment with waste drain, drain plug and overflow.

      - 14 gauge, 304 stainless steel with satin finish.

      - Rolled edges on sink rims.

      - Stainless steel legs with minimum 1 inch adjustable stainless steel bullet feet.

      - Faucet with integrated gooseneck hose sprayer.

      - Spring action type gooseneck hand-held spray hose unit with wall mount bracket to stabilize the gooseneck unit.

      - Sink backsplash to be approximately 11 inches high.

      - Sink working height from 34 to 37 inches above finished floor.

      - Accommodate .5 inch piping for hot and cold water connections.

    - Disposal:

      - Minimum .5 HP heavy duty motor.

      - Steel grinding components.

      - Appropriate controls and enclosure for safe operation.

  - Networking / Communications that shall be provided:  N / A

- Operating Options:

  - Sink:

    - _____ Replacement sink cannot be more than _____ total inches across due to limited available space.

  - Disposal: (MARK IF REQUIRED)

    - _____ Disposal required:

      - _____ Right Basin.

      - _____ Left Basin.

      - _____ Center Basin.

- Electrical Requirements:  Optional waste garbage disposal motor

# Standard U.S.A electrical specifications:  1 HP, 120 volts, 60 Hertz, 1 phase, with an average amperage draw of 10.5 amps.

- Color Requirements:  N / A

- Maintenance Requirements: Equipment contractor must have the means or support organizations in place to perform repairs on equipment.  This support is to include supervision, tools, manuals/prints, skilled labor, transportation, and parts.

  - Sustainability:  Continued parts support for equipment is required for the projected total life cycle of the equipment plus 5 years.

- Industry Standards and Contractor Requirements:

  - Industry Standards: Underwriters Laboratory (UL) listed and National Sanitation Foundation (NSF) certification, or equivalent.

  - Contractor Requirements:

    - Must be shipped ready to assemble and use.

    - No additional work shall be required on the part of the government.

# Special / Specific Equipment Safety Requirements:  N / A

- Information To Be Provided By The Contractor To The Commissary At Time of Delivery:

  - Commercial Warranty and Point of Contact for Warranty Service.

  - Installation and Operating Instructions.

  - Parts List.

# Federal Supply Class:  4510

- Special Considerations For Installation/De-installation Of Equipment: (NOTICE/WARNING)

  - Contractor:  N / A

  - Store:  N / A

- Basis of Allocation:  As required.

- Recommended Source(s) or equal:

  - Advance Tabco, 940 Series Models

  - Marlo Manufacturing Co.

- Store Request for Specific Model to Match Existing Equipment:  N / A

- Store Maintenance Requirements:  The purpose of maintenance planning is to sustain equipment operations post purchase, to identify maintenance requirements, to reduce maintenance costs, and to extend the life cycle of the equipment.

  - Within Warranty Period:

    - Consult manufacturer’s warranty documentation and contact instructions.

  - After Warranty Period:

    - Use a one time service call with store GPC as required.

    - Self-repair or replace the unit as required.