STORE NAME:				______	DODAAC:_________________

(Store name and DODAAC number to be completed when submitted to DOLM)

STORE REQUIREMENT:  _____ each

UNIT OF PACK (UPK): 1 (one) each

- Purpose of Equipment: To move palletized cargo loads from ground level to elevated storage areas.  Designed for narrow aisle applications and with a reach and tier capability.

- Generalized Operating Specifications

  - Dimensions:

    - 72 inches long, 32 inches wide. Dimensional sizes, such as length, width are not critical to this equipment, it serves as a general guide to keep this equipment sized for available space.

  - Construction:

    - Commercial off the shelf (COTS) standard unit, no unique DeCA requirements.

  - Operating Characteristics that shall be provided:

# Expected life span of this equipment is required to be 12 years or more with basic maintenance.

    - Maximum operational load rated capacity of 3000 pounds

    - Control and steering handle designed for full and easy control of travel, steering, lifting and lowering.

    - Forward and reverse.

    - Horn.

    - Provided with wide angle steering (192 degrees) to allow for right angle turns.

    - Provided with non-marking drive wheels.

    - Fork length standard at 42 inches unless otherwise specified below.

    - Battery connector Anderson (red) SB175, Pos B-20” lead.

    - Wet charged battery provided for CONUS shipment and dry charged battery for OCONUS shipment.

    - Zero degrees cold storage / freezer conditioning package standard for all OCONUS shipments.

    - Minimum 24 volt system.

    - Minimum 180 ampere hour rating.

    - Minimum 8 hours of continuous operations.

    - Control handle design provides for full and easy control of travel, steering, lift, lower, auto reverse, and horn.

# Collapsed mast unit; height not to exceed 83 inches.

# Standard elevated fork height to be 123 inches at full extension, unless otherwise specified.

# Discharge indicator and hour meter combined.

  - Networking / Communications that shall be provided: N / A

- Operating Options:

  - Standard fork tine length is 42 inches.  Optional fork length is: ____ 36 inches

  - ____ Charger required (unless this line is checked, DO NOT SEND charger).

  - Cold storage/ freezer conditioning options:

    - ____ Zero degrees cold storage / freezer conditions.

    - ____ Minus 40 degrees cold storage conditions.

  - Standard elevated fork height is 123 inches. Optional Triplex masts:

    - _____ E078 (78 inches overall height lowered, 171 inches elevated)

    - _____ E084 (84 inches overall height lowered, 189 inches elevated)

- Electrical Requirements:  Optional battery charger (CED 1R03), standard CONUS electrical requirement for charger is to be 208-480V / 60 Hz / 3 Ph (not to exceed 20 amps at 480V).  Charger to be factory set to 480 volts with the capability for adjustment at the store by an electrician as needed.

- Color Requirements:  N / A

- Maintenance Requirements: Equipment contractor must have the means or support organizations in place to perform repairs on equipment.  This support is to include supervision, tools, manuals/prints, skilled labor, transportation, and parts.

  - Sustainability:  Continued parts support for equipment is required for the projected total life cycle of the equipment plus 5 years.

- Industry Standards and Requirements:

  - Industry Standards: Underwriters Laboratory (UL) listed or equivalent.

  - Contractor Requirements:

    - Must be shipped ready to be charged and used.

    - No additional work shall be required on the part of the government.

# Special / Specific Equipment Safety Requirements:

# When the control handle is fully raised or fully lowered there is an automatic activation of brakes to stop the unit and the drive power cuts-off.

- Information To Be Provided By The Contractor To The Commissary At Time of Delivery:

  - Commercial Warranty and Point of Contact for Warranty Service.

  - Installation and Operating Instructions.

  - Parts List.

# Federal Supply Class: 3930

- Special Considerations For Installation/De-installation Of Equipment: (NOTICE/WARNING)

  - Contractor:  N / A

  - Store:  N / A

# Basis of Allocation:

# This is a one for one substitution for a 1R05-A unit, not an additional allocation.

# As required per local store requirements.

- Recommended Source(s) or equal:

  - DOD, Defense Logistics Agency (DLA) contract is DeCA’s source for MHE.

  - Yale, Model MRW030EN24TV083

- Store Request for Specific Model to Match Existing Equipment:  N / A

- Store Maintenance Requirements:  The purpose of maintenance planning is to sustain equipment operations post purchase, to identify maintenance requirements, to reduce maintenance costs, and to extend the life cycle of the equipment.

  - Within Warranty Period:

    - Consult manufacturer’s warranty documentation and contact instructions.

  - After Warranty Period:

    - Have unit added to the MHE/Baler maintenance service contract.

    - It is normal for the battery to be replaced up to two times during the life cycle of this equipment.