STORE NAME:				______	DODAAC:_________________

(Store name and DODAAC number to be completed when submitted to DOLM)

STORE REQUIREMENT:  _____ each

UNIT OF PACK (UPK): 1 (one) each

- Purpose of Equipment:

  - To move outdoor palletized cargo loads as required.

- Generalized Operating Specifications

  - Dimensions:

    - Length 146 inches with forks, 48 inches wide. Dimensional sizes, such as length, width are not critical to this equipment.  It serves as a general guide.

  - Construction:

    - Commercial off the shelf (COTS) standard unit, no unique DeCA requirements.

  - Operating Characteristics that shall be provided:

# Expected life span of this equipment is required to be 12 years or more with basic maintenance.

# Propane powered system.

    - Operational load rated capacity of 6000 pounds.

# Collapsed mast unit; height not to exceed 89 inches as two stage simplex mast or three stage triplex mast.

    - Air filled pneumatic drive and steer wheels for outdoor use.

    - Power steering, key switch, electric horn, and hour meter.

# Standard fork length is 42 inches.

# Standard elevated fork height to be 126 inches (Duplex) at full extension.

# Electronic ignition.

# Automatic transmission.

# Operator restraint system.

    - Zero degree cold storage / freezer conditioning package standard for all OCONUS shipments.

  - Networking / Communications that shall be provided:  N / A

- Operating Options:

  - Standard fork tine length is 42 inches, unless otherwise specified below.                          ____ 36 inches, ____48 inches

  - Mast option:

    - ____Simplex 86” overall height lowered – 129” elevated height.

    - ____Triplex 84” overall height lowered – 189” elevated height.

  - Cold storage option:

    - ____ To -20 degrees cold storage.

  - Cab option:

    - ____ Enclosed Operator’s cab with fans, heater, wipers, defroster.

- Electrical Requirements:  N / A

- Color Requirements:  Bright yellow.

- Maintenance Requirements: Equipment contractor must have the means or support organizations in place to perform repairs on equipment.  This support is to include supervision, tools, manuals/prints, skilled labor, transportation, and parts.

  - Sustainability:  Continued support for equipment is required for the projected total life cycle of the equipment plus 5 years.

- Industry Standards and Requirements:

  - Industry Standards: Underwriters Laboratory (UL) listed, or equivalent.

  - Contractor Requirements:

    - Must be shipped ready to be used.

    - No additional work shall be required on the part of the government.

# Special / Specific Equipment Safety Requirements:

# The operator must wear a seat belt when operating a lift truck.

# A safety platform must be used when lifting people with a lift truck.

# This propane forklift is primarily for outside use.  A propane forklift can only be used in a limited capacity as an inside forklift provided: That it is in a well ventilated area, i.e., all warehouse doors are open and good fresh air circulation is present.  For long term indoor use, an electric forklift is recommended.

- Information To Be Provided By The Contractor To The Commissary At Time of Delivery:

  - Commercial Warranty and Point of Contact for Warranty Service.

  - Installation and Operating Instructions.

  - Parts List.

# Federal Supply Class: 3930

- Special Considerations For Installation/De-installation Of Equipment: (NOTICE/WARNING)

  - Contractor:  N / A

  - Store:  This unit is intended for outdoor use only.

# Basis of Allocation:  K-1 to K-3 (one each), K-4 and K-5 (two each), when supported by mission requirements.

- Recommended Source(s) or equal:

  - DOD, Defense Logistics Agency (DLA) contract is DeCA source for MHE.

  - Yale Model No. GP060VXUAV089

- Store Request for Specific Model to Match Existing Equipment:  N / A

- Store Maintenance Requirements:  The purpose of maintenance planning is to sustain equipment operations post purchase, to identify maintenance requirements, to reduce maintenance costs, and to extend the life cycle of the equipment.

  - Within Warranty Period:

    - Consult manufacturer’s warranty documentation and contact instructions.

  - After Warranty Period:

    - Have unit added to the MHE/Baler maintenance service contract.