# Purpose of this Equipment:  To compress cardboard and plastic into bales to reduce scrap volume and for recycling.

- Federal Supply Class:  3540

- General Operating Specifications:

  - Dimensions:

    - Overall unit height limited to a maximum of 14 feet (168 inches).

  - Salient Characteristics that shall be provided:

    - Various system performance specifications, such as, platen force and system pressures are acceptable so long as the bale requirements are met.

    - Approximate bale dimensions: 72 inches wide, 30 inches deep, 48 inches high.

    - Vertical style baler.

    - Self-contained hydraulic system.

    - Bale press to be all steel construction.

    - Front loading with the loading door opening on the upper front face of the baler.  Minimum height on feed opening is 25 inches by bale length 70 inches.

    - Minimum 10 HP motor.

    - Automatic bale ejection.

    - Easy tie wire platens with a minimum of 6 slots for wire insertion.

    - Automatic start / compress / stop operating sequence.

    - All control circuitry contained in NEMA 4 enclosures.

    - Door to be right or left hand hinged as specified.

    - Tri-voltage 208/230/460V 3 phase multi-tap transformer.

    - Interlock system to prevent operation when doors are open and to ensure proper operational sequencing.

    - Moving parts are shielded or enclosed during operations.

    - Key operated start switch.

    - Pressure relief valve on the hydraulic system to prevent over pressurization.

# Industry Standards / Requirement(s):

    - Underwriters Laboratory (UL) listed or equivalent. CE is not acceptable for a standard.

    - Warranty period shall be for 1 year parts and labor, 3 years on major components and 10 years on welds and frames.

  - Information Technology requirements:  N / A

# Color Requirement(s):  N / A

- Electrical Requirements:

# Standard U.S.A. electrical specifications:  208 volts, 60 Hertz, 3 phase.

# _____ Pacific Theater stores 208 V / 50 HZ / 3 PH (Applies to these stores only: Atsugi, Misawa, Sagami Depot, Sagamihara, Yokosuka, Yokota, Camp Zama, and Kanto Plains CDC. All other stores are standard U.S.A. 60 cycle voltages.)

# Standard Contractor Requirements:

  - Must be shipped ready to assemble, mount, and make utility connections and use.

  - De-installation of old equipment is required.

  - This equipment requires disposal.  Contractor to remove from commissary and dispose.

  - Installation required by an authorized service technician.

  - Operational testing is required.

  - Startup assistance required.

  - Provide familiarization training.

  - Information to be provided by the Contractor to the commissary at the time of delivery:

    - Point of Contact for Service

    - Installation and Operating Instructions

    - Parts List

  - Maintenance Sustainability Requirements: Continued parts support is required for the projected life cycle of the equipment plus 5 years.

# Special Coordinating / Safety Instructions:

  - Contractor:  The unit being replaced is to be removed from the premises.

  - Store:

    - Supplies must be ordered such as baling wire and crimpers for this unit.  These items are not furnished with the unit.

    - The electrical feed to the baler must be hardwired.

    - Medium baler, 1R00-B weighs approximately 8000 lbs. Store may have to borrow MHE from installation, or have vendor bring MHE to move the unit.

    - MUST BE COMPLETED WHEN REQUESTING BALER REPLACEMENT:

      - Why is a new baler being requested?

        - _____Recycle Program

        - _____Replacement

        - _____Construction Project

        - _____ Other: ______________________

      - Door hinges: _____ Right or _____Left

      - Will this be a new installation?  ____ Yes  ____ No

      - If Yes, is there electrical power box adjacent to the location?  ____ Yes  ____ No

      - If yes, what is the voltage?  ____460v ____230v ____ 208v

      - Will this replace a pre-existing baler?  ____ Yes  ____ No

      - If Yes, existing Model: _________________________

      - If Yes, existing Make:  _________________________

      - Does this baler belong to DeCA?  ____ Yes  ____ No

      - If Yes, what is the bar code? ______________________

      - Is the current electrical disconnect box located:

        - _____ on the wall

        - _____ on the baler

STORE ORDER REQUEST

STORE NAME: __________________________    DODAAC: ____________________

QUANTITY: _______